Nuclear Instrumentation and Measurement

0407204, Fall 2024-25

Midterm Exam

| Instructor: | Dr. Muhammad Zubair |
| --- | --- |
| CRN: | 11582 |
| Section: | 11 |
| Date and Time: | October 14, 2024, 2:00– 3:00 PM |
| Place: | M10-004 |

| Student Name: |  | ID: |  |
| --- | --- | --- | --- |

Grading

| Q1 | Q2 | Q3 | Total |
| --- | --- | --- | --- |
| 10 | 10 | 10 | 30 |

Useful Data:

1 amu=1.6605x10-27 kg, 1 amu  931.48 MeV, 1 eV = 1.6022  10-19 J , me = 0.0005486 amu

235U = 235.0439, 117Sn = 116.9029, 118Sn = 117.9016

Answer all of the following questions:

Question #1 (5+5 Points):

- For each of the radiation sources listed below, indicate whether "discrete" or "continuous" is a better description:

- Conversion electrons

- Auger electrons

- Fission fragments

- Bremsstrahlung

- Annihilation radiation

Discrete: i, ii, v

Continuous: iii, iv

- Using the reaction below, find the energy released by the spontaneous fission of 235U into two equal-mass fragments.

235U  117Sn + 118Sn

Question #2 (5+5):

- Calculate the mean free path of 1 MeV gamma rays in sodium iodide (specific gravity= 3.67, and the mass attenuation coefficient is 0.06 cm2/gm).

- What fraction of 2-MeV betas will go through a single Al (density = 2.7x103 kg/m3) foil of thickness 0.1 mm? Where; μ = 0.7714 m2/Kg

Question #3 (5+5 points):

- A 10-min count of a source+ background gives a total of 846 counts. Background alone counted for 10 min gives a total of 73 counts. What is the net counting rate due to source alone, and what is its associated standard deviation?

- Explain in your words the term, “Minimum Detectable Activity”.

The minimum detectable activity (MDA) is the smallest net count that can be reported with a certain degree of confidence that represents a true activity from a sample and is not a statistical variation of the background.