Department of Mechanical and Nuclear Engineering

Nuclear Reactor Theory

0407308

Final Examination Solution

Spring 2023-2024

Time/Date:  8:30 - 10:30, May 8, 2024

Room: W10-002

Instructor: Dr. Thanh Mai Vu

| Student Name | Student ID |
| --- | --- |

Rules:

- Begin by writing your name and student ID.

- Closed-book exam, only a calculator is allowed.

- Using mobile phone and smart watch during exam is not allowed. Any academic dishonesty will result in “F” for your final grade.

- Write the answer neatly and clearly. The result should be written with its unit.

For Instructor:

| Question | 1 | 2 | 3 | 4 | 5 | 6 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Grade |  |  |  |  |  |  |  |
| Out of | 6 | 6 | 6 | 6 | 15 | 6 | 45 |

Useful Equations:

One group NDE:

-D∇2ϕr+Σaϕr=1kνΣfϕr

keff=νΣfDB2+Σa

PNL=11+B2L2

D=13Σtr

Material Buckling

Bm2=νΣf-ΣaD

The leakage to absorption ratio=DB2Σa

For cylindrical geometry:

B2=2.405R2+πH2

ϕr,z=AJ02.405RrcosπHz

The average neutron flux in the reactor.

ϕavg=PERΣfV

ϕmaxϕavg=3.64

Problems:

- (Total 6 points)

a. (2 points) What is reaction rate?

Solution:

The reaction rate is the number of interactions taking place in one cubic centimeter per one second.

- b. (4 points) A monoenergetic beam of neutrons,ϕ = 4  1010 neutrons/cm2-sec, impinges on a target 1 cm2 in area and 0.1 cm thick. There are 0.048 x 1024 atoms per cm3 in the target, and the total cross-section at the energy of the beam is 4.5 b.

Calculate the macroscopic total cross-section and number of neutron interactions per second occur in the target.

Solution:

- The macroscopic total cross-section:

Number of reaction=tV=8.64*108(interactions)

Number of neutron interactions per second occur in the target:

- (Total 6 points)

a. (3 points) Write the 4-factor formula and define each factor

Solution:

Four factor formula

k∞=ηpfε

b. (3 points) In a thermal nuclear reactor at the beginning of its life for every 1,000 neutrons, 500 neutrons are absorbed in 235U, 225 neutrons are absorbed in 238U, 125 neutrons are absorbed in coolant and cladding, 150 neutrons leak out from the geometrical core boundaries, and ν is 2.43.

Calculate the multiplication factor and the conversion ratio value for this reactor

Solution:

Assume that all neutrons absorbed in U235 cause fission reaction, the multiplication factor of the reactor is:

k1  =500*2.43/1000=1.215

The conversion ratio value for this reactor?

CR=Fissile Material ProducedFissile Material Destroyed=225500=0.45

- (Total 6 points)

Leakage

Source

a. (3 points) Complete the following neutron transport equation:

1v∂ϕ∂t+                                                     +Σtr,Eϕr,E,Ω, t=

+

In-scattering

Solution:

1v∂ϕ∂t+Ω∙∇ϕ(r,E,Ω,t) +Σtr,Eϕr,E,Ω, t

=4πdΩ'0∞dE'ΣsE'→E,Ω'→Ωϕr,E',Ω', t+sr,E,Ω,t

b. (3 points) Three isotropic neutron sources, each emitting S neutrons/sec, are located in an infinite vacuum at the three corners of an equilateral triangle of side a.

Find the flux at P1 - the midpoint of one side.

Flux of point source in infinite medium:

ϕr=S4πDre-r/L

Solution:

Total flux at position P1:

ϕP1=2×S4πDa2  e-a2L+S4πD3a2e-3a2L=SπDae-a2L+e-3a2L23

- (Total 6 points) A plane source of strength So neutrons/sec is placed in an infinite medium. The material has an absorption cross section Σa and a diffusion coefficient D. Neglect the extrapolation distance.

  - (3 points) Write down the diffusion equation and its general solution for this problem for

x≠0.

Solution:

L2=DΣa

∇2ϕx-1L2ϕx=-SxD

→d2ϕ(x)dx2-1L2ϕx=0,  x≠0

General solution: ϕ(x)=Ae-x/L+Cex/L

  - (3 points) Solve the general equation for ϕ(r) by applying 2 following conditions:

limx→∞ϕ(x)=0

limx→0Jx=S02

Solution:

Applying physical condition:

limx→∞ϕ(x)=0

limx→∞Ae-x/L+Cex/L=0, then C=0

∴ϕ(x)=Ae-x/L

Applying source condition:

limx→0J(x)=S02

Determine A: From Fick’s law:

Jx=-Ddϕdx=-DdAe-xLdx

Jx=DALe-xL,  x>0

Then

limx→0J(x)=limx→0DALe-xL=DAL=S02→A=S0L2D

∴ϕx=S0L2De-xL,  x>0

Final solution: ϕx=S0L2De-|x|L

- (Total 15 points)

For a cylindrical shape reactor with the following information:

- Initial boron concentration = 2200 ppm

- σa of Boron = 600 b,       σtr of Boron = 0 b.

- Mass number of boron = 10.811

- For water, Σa=0.035 /cm, Σf=0.00 /cm, Σtr=0.050 /cm

- For fuel,     Σa=0.650 /cm, Σf=0.40 /cm, Σtr=0.040 /cm,    υ=2.5

- Fuel to Moderator volume ratio= 1/3

- Avogadro number = 6.022x1023 atom/mole

- Power=1000 MWth

- Do not neglect the extrapolation distance.

Answer the following equation using 1-group diffusion theory:

  - (2 points) Calculate Σat for a mixture of fuel, water, and boron.

NB=2200 g106 g=2.2×10-310.811 g/mole×0.6022×1024atom/mole

NB=1.225×1020atomg≈1.225×10-4atomb cm

ΣaB=NBσa=0.074 cm-1

Σa=0.244 cm-1

  - (2 points) Calculate the diffusion coefficient.

D=13Σtr=7.092 cm

  - (2 points) Find a geometrical buckling that makes the reactor critical.

Bg2=Bm2=νΣf-ΣaD=8.699×10-4 cm-2

  - (3 points) If the diameter of the reactor is 150 cm, calculate the critical height of cylinder.

d=0.71×3D=14.947

Br2=2.405R+d2=7.149×10-4 cm-2

Bm2=Bg2=Br2+Bz2

Bz2=1.550×10-4 cm-2

H=πBz-2d=222.462 cm

  - (2 points) Calculate the leakage to absorption ratio.

=DB2Σa=0.103

  - (2 points) Calculate the average neutron flux in the reactor.

ϕavg=PERΣfV=7.949×1013 neutrons/cm2s

  - (2 points) Calculate the maximum neutron flux in the reactor.

ϕmax=ϕavg×3.64=2.894×1014 neutrons/cm2s

- (Total 6 points) Diffusion equation for group g is written as follows:

1vg∂ϕgr,t∂t-∇⋅Dgr∇ϕgr,t  +Σtgrϕgr,t

=  χgg'=1GνΣfg'(r)ϕg'(r,t)   +g'=1G Σsg'→g(r)ϕg'(r,t)+ Sg(r,t)

a. (3 points) Explain the meaning of each term of the equation

b. (2 points) Write down the multigroup diffusion equation for steady-state condition.

c. (1 point) Write down the steady-state multigroup diffusion equation in case the fission source is much more dominant compared to the external source.

Solution:

Leakage

Total interaction

Fission source

Scattering in

External source

Rate of change in neutron flux

a.

1vg∂ϕgr,t∂t-∇⋅Dgr∇ϕgr,t  +Σtgrϕgr,t

=  χgg'=1GνΣfg'(r)ϕg'(r,t)   +g'=1G Σsg'→g(r)ϕg'(r,t)+ Sg(r,t)

b. For steady-state condition:

-∇⋅Dg(r)∇ϕg(r)  +Σtg(r)ϕg(r) =  χgg'=1GνΣfg'(r)ϕg'(r)   +g'=1G Σsg'→g(r)ϕg'(r)+ Sg(r)

c. Neglecting the external source:

-∇⋅Dg(r)∇ϕg(r)  +Σtg(r)ϕg(r) =  χgg'=1GνΣfg'(r)ϕg'(r)   +g'=1G Σsg'→g(r)ϕg'(r)